TE Connectivity’s MHP-TAM Series Offers Compact Thermal Protection for Lithium Batteries Used in Ultra-Thin Portable Consumer Electronics Resettable TCO device helps designers meet high peak-current requirements Shanghai – March 17, 2015 – Battery applications designers are tasked with meeting increasingly stringent safety standards for ultra-thin notebooks and other portable consumer electronics powered by high-capacity lithium polymer and prismatic cells. In response, TE Connectivity’s business unit, TE Circuit Protection, introduces the new MHP-TAM series. The MHP-TAM device features an ultra-low-profile (L: 5.8mm x W: 3.85mm x H: 1.15mm max.) package and a high (9VDC) rating. The devices in the series offer a choice of two different levels of current-carrying capacity and multiple cut-off temperature ratings. The device provides a space-efficient thermal cutoff (TCO) solution that helps designers meet the demanding peak-current requirements of consumer products. Utilizing innovative MHP (metal hybrid PPTC) technology, the MHP-TAM device combines a bimetal protector in parallel with a PPTC (polymeric positive temperature coefficient) device. In battery applications, the MHP-TAM device helps provide resettable over-temperature protection to shut down the battery when a fault is detected. The device rests when the fault is removed. The devices in the MHP-TAM series feature different open temperatures, ranging from 72°C to 90°C (typical), which are appropriate for the battery market. They also offer two levels of hold currents: low-current (approximately 6A at 25°C) and high-current (approximately 15A at 25°C). “To help meet the thermal protection requirements of lithium polymer battery applications, our new MHP-TAM devices have been tested to withstand the harshest humidity test conditions in the battery industry. High reliability is important since OEMs are tightening their component reliability requirements for lithium polymer battery packs used in consumer applications,” said Amy Chu, Product Manager of TE Circuit Protection. “Over the last 30 years, TE Circuit Protection has been a front-runner in utilizing PPTC technology to introduce high-reliability solutions for battery protection. The addition of the new MHP-TAM series offers designers even more choices when selecting the best protection device for today’s consumer electronics.” The MHP-TAM series complies with the regulatory agency requirements of the most rigorous international quality and safety standards: UL, cUL and CB. Price: MHP-TAMxx-9-xx – Available upon request Availability: Now Delivery: 6-8 weeks ARO For further information, please visit TECP online at www.circuitprotection.com. ABOUT TE CONNECTIVITY TE Connectivity (NYSE: TEL) is a $14 billion global technology leader. Our connectivity and sensor solutions are essential in today's increasingly connected world. We collaborate with engineers to transform their concepts into creations – redefining what's possible using intelligent, efficient and high-performing TE products and solutions proven in harsh environments. Our 80,000 people, including 7,500 design engineers, partner with customers in over 150 countries across a wide range of industries. We believe EVERY CONNECTION COUNTS – www.TE.com. # # #. Contacts:; Media Relations: Matthew Bucklin TE Connectivity (650) 361-2089 matthew.bucklin@te.com; Media Relations: Amy Chu TE Connectivity (86) 21 6106 7364 amy.chu@te.com.. TE Connectivity, TE and TE connectivity (logo) are trademarks of the TE Connectivity Ltd family of companies.